# E-Series Resistor Calculator

User Manual

# 1. Purpose

Determines the nearest standard resistor value from a selected IEC E-series.

Displays both the nearest value and percentage error.

# 2. Supported Series

E3, E6, E12, E24 → 2 significant digits

E48, E96, E192 → 3 significant digits

# 3. Input Format

Plain numbers: 470, 4.7, 0.0047

Optional Exponential: 4.7e2, 1E-3

Optional single suffix letter: 470k, 4.7u. Suffix has no effect on numeric calculations.

# 4. Exponent Handling

If exponent is used in input, it is preserved exactly in output.

Case is preserved (e vs E).

# 5. Output Rules

Correct number of significant digits based on selected series.

Suffix letters are preserved but not used in calculations.

# 6. Error Calculation

Error = 100 × (Nearest / Input − 1) %

# 7. How the Nearest Value is Selected

1. The input is normalized to the form m × 10^n, where 1 ≤ m < 10.

2. The program compares m against all normalized values in the selected E-series.

3. It computes percentage error: |(candidate / m) − 1|.

4. The value with the smallest absolute error is selected.

5. The selected normalized value is scaled back using 10^n.

6. A comparison with 10.0 ensures correct rollover to the next decade.

# 8. Examples

Input: 456 (E12) → Output: 470

Input: 4.56 (E12) → Output: 4.7

Input: 456 (E96) → Output: 453

Input: 4.56e2 (E96) → Output: 4.53e2

# 9. Notes

Suffix letters are cosmetic only.

Exponent is not normalized.

Selection is based on minimizing percentage error, not absolute difference.